Glossary

AHU	Air handling unit.

ASHRAE 	American Society of Heating, Refrigerating and Air-conditioning Engineers, Inc.

BTU	British thermal unit.

Building Related Illness 	Illnesses that can be traced to specific bacteria, airborne pollutants, or toxic substances found in the building. Illnesses are diagnosable, for example, Legionnaire's disease and hypersensitivity pneumonitis. (Contrast Sick Building Syndrome)

CFM	Cubic feet per minute (volume).

Chilled beam	A cooling element situated above a ceiling that cools using convection and employs natural or induced air flows.

Combustion Efficiency	Ratio of heat obtained from the combustion to the theoretical heat content of the fuel.

Condensate	Liquid water condensed from vapor.

Condensing boilers	A condensing boiler utilizes the latent heat of water produced from the burning fuel, in addition to the standard sensible heat, to increase its efficiency.

Demand Control Ventilation 	A DCV adjusts outside ventilation air based on the number of occupants and the ventilation demands that those occupants create. This strategy reduces the amount of unnecessary outside air conditioning.

Efficiency	The ratio of energy output to energy input of a process or machine.

Energy Recovery Wheel	Also called an enthalpy wheel, recovery wheels can transfer both heat (sensible energy) and moisture (latent energy).

Energy Star 	Voluntary partnership between business and government that provides a framework to promote environmental and financial benefits of improved energy performance

First Cost				Considers the cost of new equipment and installation.

Ground source heat pump	Central  HVAC System that uses the Earth as a heat source (winter) or heat sink (summer).

Heat exchanger	A device for transferring heat from one medium to another.

Heat pipe	Energy recovery systems that uses a refrigerant as the working fluid, it is able to transfer heat between two air streams.

Heat pump	Vapor compression cycle refrigeration system that reverses the flow of refrigerant to extract heat from an existing low 	intensity source and “pump” it to useful high intensity values.

HEPA	High Efficiency Particulate Arrestance air filters.

HSPF	Heating Season Performance Factor is a measure of the overall heating efficiency of a heat pump.

Human thermal comfort	The state of mind that expresses satisfaction with the surrounding environment (ANSI/ASHRAE Standard 55).

IEQ	Indoor environmental quality.

IAQ	Indoor air quality.

Latent energy	The amount of energy released or absorbed during a change of state that occurs without a change in temperature, meaning a phase transition such as water changing into vapor. Latent energy recovery transfers both temperature and humidity.

Life cycle cost	Considers the total cost of owning, operating and maintaining equipment and infrastructure over its useful life.

LEED 	Leadership in Energy and Environmental Design

Mixed-Mode (MM)	A combination of natural ventilation and mechanical cooling.

Natural ventilation (NV)	Flow of outdoor air caused by wind and thermal pressures through intentional openings in the building’s shell.

Primary air	The air which is mixed, during combustion, with the fuel just before the point of combustion.

Plenum	In UFAD systems, the raised floor assembly that is integrated with an HVAC system.

PMV	Predictive Mean Vote. An equation for measuring the thermal comfort level of a large group of people occupying the same space.

PPD	Predictive Percentage Dissatisfied. Index predicting dissatisfaction of a body of people regarding their thermal environment.

Psychrometrics	The graphical study of the conditions of air pertaining to moisture, temperature, and total heat content. The psychrometric chart gives the relative humidity for a given wet-bulb and dry-bulb temperature. It can be used 	for sizing air conditioning equipment. To use, find the vertical line for the dry bulb temperature at the bottom of the chart, and the slanted line for the wet-bulb temperature at the curved top-left edge. Find the intersection point of these two lines, and read the percent relative humidity from the curved lines nearest that point.

Refrigerant	Chemical used to absorb and transfer heat (latent heat transfer).

Refrigeration	The process of absorbing heat from a location where it is not wanted and moving it to a location where it is discarded.

Runaround coil loop	Energy recovery device that consists of a coil that transfers sensible heat between two or more air streams.

Secondary air	Additional air supplied after the fuel and primary air mixture to ensure complete combustion and excess air.

Sensible energy	The heat absorbed or transmitted by a substance during a change of temperature which is not accompanied by a change of state. Sensible energy recovery refers to transferring only heat between fluids, no moisture.

Setpoint	The desired conditions from the controller.

Sick Building Syndrome (SBS)	A set of symptoms that affects some building occupants during the time they spend in a particular building, but symptoms typically disappear when they leave the building. (Contrast Building Related Illness)

Stack effect	The air flow that results from warm air rising. This effect can overpower mechanical ventilation systems and disrupt ventilation and air circulation in a building.

Stratification	A separation of air into lower and upper zones caused by temperature and pressure differentials in a room. The lower zone has cooler and fresher air, and the upper zone has warmer and more polluted air.

Thermal decay	In UFAD systems, it is the difference between the temperature of the primary supply air entering the plenum through plenum inlets and the temperature of the air leaving the plenum through floor diffusers, or other terminal devices.

Thermal Energy Storage (TES)    Building cooling system that cool a storage medium and then use that cold medium to cool air or water at a later point in time.

Total Cost of Ownership (TCO)  Includes the initial costs to implement a project together with the continuing costs to maintain, modify, train staff, house, deploy, provide infrastructure or any other cost associated with the project, including final decommissioning. TCO is an estimate that includes all direct and indirect costs over the useful life of the application, which is commonly used in full cost accounting systems.

Underfloor air distribution 	Heating and ventilation delivered through raised floor plenums, especially in areas with tall ceilings, through ducts in floor cavities.

Variable Air Volume (VAV) 	Air handling system that conditions the air to constant temperature and varies the outside airflow to ensure thermal comfort. Air flow is varied to match heating and cooling loads.

Variable Frequency Drive (VFD) Automated speed control for electric motors like fans, pumps, and centrifugal chillers for precise control and energy savings.

Variable Speed Drive (VSD)	See VFD.